|  | Customs Department under the Ministry of Finance of the Republic of Lithuania |  |
| --- | --- | --- |
| # CUSTOMS CONTROL SYSTEM FOR PRESENTATION OF GOODS # Interface Specification for customs clients # Reference PPMKS-CLIENT-IS-EN # Version 1.00 # Volume II Presentation Notification |  |  |

# Table of contents

1	Introduction	4

2	Message flows	5

2.1	Presentation Notification	5

2.1.1	Core flow	5

2.1.2	Amendment	7

3	Messages	8

3.1	IEG330	9

3.2	IEG347	12

3.3	IEG348	18

3.4	IEG349	23

3.5	Rules	26

# List of figures

Figure 1 Presentation Notification	5

# Introduction

See volume I for full content of introduction and common information for all volumes.

This volume covers definitions of the messages and message flows of the following declarations/notifications:

- Presentation Notification (messages IEG3xx),

# Message flows

## Presentation Notification

### Core flow

Figure 1 Presentation Notification

Person Presenting the Goods (Trader) prepares Presentation Notification and sends it to Customs Office of Presentation (PPMKS system) as IEG347 message.

PPMKS validates it against the rules defined in PPMKS-CLIENT-IS specification including existence/valid state of pointed ENSes in EU Common Repository (for diagram's clarity not depicted here).

In case of errors found to the trader the rejection is send and flow is finished here. Rejection is sent as IEG348 message with error details.

In case of lack of errors [Validation OK] the flow is continued as follows.

PPMKS registers Presentation Notification and assigns MRN. To the trader acceptance message IEG348 with MRN is sent.

Now on pointed by Presentation Notification ENSes, or their parts if only parts are pointed, the control processing is run. It can be real goods/documents examination or decision that goods are considered satisfactory without real control..

In case of discrepancies found the presentation notification items rejection message IEG349 is prepared and sent to trader - flow is finished here.

Otherwise (no discrepancies), the System prepares the IEG330 finalization message and sends it to trader. Process finishes here.

### Amendment

Person Presenting the Goods, till PN finalization, can send amendment. This is IEG347 with MRN instead LRN.

The process is exactly the same as Core flow but MRN is not generated again. If such amendment message is validated by PPMKS with success then it will replace previously sent Presentation Notification.

# Messages

This page is intentionally empty (each message definition starts from new page).

## IEG330

PRESENTATION NOTIFICATION ITEMS FINALIZATION

This message is sent to the Person Presenting the Goods from Customs Office of Presentations to inform for what Entry Summary Declarations presentation is finalized (and with what result - see ControlResult).

Structure

| IEG330 | 1 | - |
| --- | --- | --- |
| — Notification | 1 | - |
| — — Declaration | 1..9999 | - |
| — — — ControlResult | 1 | - |
| — — PersonPresentingGoods | 1 | - |
| — — CustomsOfficeOfPresentation | 1 | - |
| — Signature | 0..1 | - |

Contents

IEG330

| Root element of the message | - |  |  | 1 | - |
| --- | --- | --- | --- | --- | --- |

| preparationDateAndTime Preparation date and time of the message | - | dateTime | - | R | - |
| --- | --- | --- | --- | --- | --- |
| messageIdentification Message identification Unique identifier of the message assigned by sender. | - | an..40 | - | R | R783LT |

IEG330.Notification

| Notification G3 declaration (Presentation Notification) header data level | - |  |  | 1 | - |
| --- | --- | --- | --- | --- | --- |

| mrn MRN Master Reference Number (MRN) of Presentation Notification. | - | an18 | - | R | R780LT R788LT |
| --- | --- | --- | --- | --- | --- |

IEG330.Notification.Declaration

| Entry Summary Declaration Fxx declaration header data level | - |  |  | 1..9999 | - |
| --- | --- | --- | --- | --- | --- |

| mrn MRN Master Reference Number (MRN) | - | an18 | - | R | R780LT R788LT |
| --- | --- | --- | --- | --- | --- |

IEG330.Notification.Declaration.ControlResult

| ControlResult | - |  |  | 1 | - |
| --- | --- | --- | --- | --- | --- |

| code Code | - | an..2 | 328COM | R | - |
| --- | --- | --- | --- | --- | --- |
| date Date | - | date | - | R | - |
| description Description | - | an..1024 | - | O | R780LT |

IEG330.Notification.PersonPresentingGoods

| Person presenting the goods | 13 11 000 000 |  |  | 1 | - |
| --- | --- | --- | --- | --- | --- |

| identificationNumber Identification number | 13 11 017 000 | an..17 | - | R | R790LT R806LT |
| --- | --- | --- | --- | --- | --- |

IEG330.Notification.CustomsOfficeOfPresentation

| Customs office of presentation | 17 09 000 000 |  |  | 1 | - |
| --- | --- | --- | --- | --- | --- |

| referenceNumber Reference number | 17 09 001 000 | an8 | COL | R | R795LT R804LT |
| --- | --- | --- | --- | --- | --- |

IEG330.Signature

| Digital signature See chapter "Digital signature". | - |  |  | 0..1 | - |
| --- | --- | --- | --- | --- | --- |

## IEG347

PRESENTATION NOTIFICATION

Presentation of goods to customs. See column G3 of UCC-DA, Annex B. It is sent by Person presenting goods to Customs office of presentation.

To change an already sent notification, the MRN must be provided instead of the LRN.

Structure

| IEG347 | 1 | - |
| --- | --- | --- |
| — Notification | 1 | - |
| — — Declaration | 1..9999 | - |
| — — — Consignment | 1 | - |
| — — — — TransportDocumentMasterLevel | 0..1 | R803LT |
| — — — — Carrier | 0..1 | R805LT |
| — — — — LocationOfGoods | 1 | - |
| — — — — — CustomsOffice | 0..1 | R807LT |
| — — — — — GNSS | 0..1 | R807LT |
| — — — — — EconomicOperator | 0..1 | R807LT |
| — — — — — Address | 0..1 | R807LT |
| — — — — — PostcodeAddress | 0..1 | R807LT |
| — — — — TransportEquipment | 0..9999 | - |
| — — — — Receptacle | 0..999 | - |
| — — — — HouseConsignment | 0..99 | R803LT |
| — — — — — TransportDocumentHouseLevel | 0..1 | R803LT |
| — — PersonPresentingGoods | 1 | - |
| — — CustomsOfficeOfPresentation | 1 | - |
| — Signature | 0..1 | R796LT R797LT R798LT |

Contents

IEG347

| Root element of the message | - |  |  | 1 | - |
| --- | --- | --- | --- | --- | --- |

| messageSender Message sender Technical data item pointing identification number (XPath notation) of a sender of the message. Possible values: /IEG347/Notification/PersonPresentingGoods/identificationNumber | - | an..255 | - | R | R785LT R786LT R802LT |
| --- | --- | --- | --- | --- | --- |
| preparationDateAndTime Preparation date and time of the message | - | dateTime | - | R | - |
| messageIdentification Message identification Unique identifier of the message assigned by sender. | - | an..40 | - | R | R783LT R784LT |

IEG347.Notification

| Notification G3 declaration header data level | - |  |  | 1 | - |
| --- | --- | --- | --- | --- | --- |

| lrn LRN Local Reference Number (LRN) of Presentation Notification. | 12 09 000 000 | an..22 | - | O | R787LT R791LT R800LT R801LT |
| --- | --- | --- | --- | --- | --- |
| mrn MRN Master Reference Number (MRN) of Presentation Notification. | - | an18 | - | O | R780LT R788LT R800LT R801LT |
| declarationDate Declaration date Presentation Notification (or its amendment) issuing date and time | 15 06 000 000 | dateTime | - | R | R793LT |

IEG347.Notification.Declaration

| Entry Summary Declaration Fxx declaration header data level | - |  |  | 1..9999 | - |
| --- | --- | --- | --- | --- | --- |

| mrn MRN Master Reference Number (MRN) | - | an18 | - | R | R780LT R788LT |
| --- | --- | --- | --- | --- | --- |

IEG347.Notification.Declaration.Consignment

| Master Consignment data level | - |  |  | 1 | - |
| --- | --- | --- | --- | --- | --- |

IEG347.Notification.Declaration.Consignment.TransportDocumentMasterLevel

| Transport document | 12 05 000 000 |  |  | 0..1 | R803LT |
| --- | --- | --- | --- | --- | --- |

| referenceNumber Reference number | 12 05 001 000 | an..70 | - | R | R780LT |
| --- | --- | --- | --- | --- | --- |
| type Type | 12 05 002 000 | an4 | 329COM | R | - |

IEG347.Notification.Declaration.Consignment.Carrier

| Carrier | 13 12 000 000 |  |  | 0..1 | R805LT |
| --- | --- | --- | --- | --- | --- |

| identificationNumber Identification number | 13 12 017 000 | an..17 | - | R | R790LT R806LT |
| --- | --- | --- | --- | --- | --- |

IEG347.Notification.Declaration.Consignment.LocationOfGoods

| Location of goods | 16 15 000 000 |  |  | 1 | - |
| --- | --- | --- | --- | --- | --- |

| typeOfLocation Type of location | 16 15 045 000 | an1 | 302COM | O | - |
| --- | --- | --- | --- | --- | --- |
| qualifierOfIdentification Qualifier of identification | 16 15 046 000 | an1 | 276COM | O | - |
| unLocode UN/LOCODE | 16 15 036 000 | an..17 | 353COM | O | R807LT |
| authorisationNumber Authorisation number | 16 15 052 000 | an..35 | - | O | R780LT R807LT |
| additionalIdentifier Additional identifier | 16 15 053 000 | an..4 | - | O | R780LT R808LT |

IEG347.Notification.Declaration.Consignment.LocationOfGoods.CustomsOffice

| Location of goods, Customs office | 16 15 047 000 |  |  | 0..1 | R807LT |
| --- | --- | --- | --- | --- | --- |

| referenceNumber Customs office reference number | 16 15 047 001 | an8 | COL | R | - |
| --- | --- | --- | --- | --- | --- |

IEG347.Notification.Declaration.Consignment.LocationOfGoods.GNSS

| Location of goods, GNSS | 16 15 048 000 |  |  | 0..1 | R807LT |
| --- | --- | --- | --- | --- | --- |

| latitude Latitude | 16 15 048 049 | an..17 | - | R | - |
| --- | --- | --- | --- | --- | --- |
| longitude Longitude | 16 15 048 050 | an..17 | - | R | - |

IEG347.Notification.Declaration.Consignment.LocationOfGoods.EconomicOperator

| Location of goods, Economic operator | 16 15 051 000 |  |  | 0..1 | R807LT |
| --- | --- | --- | --- | --- | --- |

| identificationNumber Identification number | 16 15 051 017 | an..17 | - | O | R790LT R806LT |
| --- | --- | --- | --- | --- | --- |

IEG347.Notification.Declaration.Consignment.LocationOfGoods.Address

| Location of goods, Address | 16 15 018 000 |  |  | 0..1 | R807LT |
| --- | --- | --- | --- | --- | --- |

| streetAndNumber Street and number | 16 15 018 019 | an..70 | - | R | R780LT |
| --- | --- | --- | --- | --- | --- |
| postcode Postcode | 16 15 018 021 | an..17 | - | R | R780LT |
| city City | 16 15 018 022 | an..35 | - | R | R780LT |
| country Country code | 16 15 018 020 | an2 | 356COM | R | - |

IEG347.Notification.Declaration.Consignment.LocationOfGoods.PostcodeAddress

| Location of goods, Postcode Address | 16 15 081 000 |  |  | 0..1 | R807LT |
| --- | --- | --- | --- | --- | --- |

| postcode Postcode | 16 15 081 021 | an..17 | - | R | R780LT |
| --- | --- | --- | --- | --- | --- |
| houseNumber House number | 16 15 081 025 | an..35 | - | O | R780LT |
| country Country | 16 15 081 020 | an2 | 356COM | R | - |

IEG347.Notification.Declaration.Consignment.TransportEquipment

| Transport Equipment | 19 07 000 000 |  |  | 0..9999 | - |
| --- | --- | --- | --- | --- | --- |

| containerIdentificationNumber Container identification number | 19 07 063 000 | an..17 | - | R | R794LT |
| --- | --- | --- | --- | --- | --- |

IEG347.Notification.Declaration.Consignment.Receptacle

| Receptacle | - |  |  | 0..999 | - |
| --- | --- | --- | --- | --- | --- |

| receptacleIdentificationNumber Receptacle identification number | 19 11 000 000 | an..35 | - | R | R780LT |
| --- | --- | --- | --- | --- | --- |

IEG347.Notification.Declaration.Consignment.HouseConsignment

| House Consignment data level | - |  |  | 0..99 | R803LT |
| --- | --- | --- | --- | --- | --- |

| sequenceNumber Sequence number As stated in ENS. | - | n..5 | - | R | R781LT |
| --- | --- | --- | --- | --- | --- |

IEG347.Notification.Declaration.Consignment.HouseConsignment.TransportDocumentHouseLevel

| Transport document | 12 05 000 000 |  |  | 0..1 | R803LT |
| --- | --- | --- | --- | --- | --- |

| referenceNumber Reference number | 12 05 001 000 | an..70 | - | R | R780LT |
| --- | --- | --- | --- | --- | --- |
| type Type | 12 05 002 000 | an4 | 329COM | R | - |

IEG347.Notification.PersonPresentingGoods

| Person presenting the goods | 13 11 000 000 |  |  | 1 | - |
| --- | --- | --- | --- | --- | --- |

| identificationNumber Identification number | 13 11 017 000 | an..17 | - | R | R790LT R802LT R806LT |
| --- | --- | --- | --- | --- | --- |

IEG347.Notification.CustomsOfficeOfPresentation

| Customs office of presentation | 17 09 000 000 |  |  | 1 | - |
| --- | --- | --- | --- | --- | --- |

| referenceNumber Reference number | 17 09 001 000 | an8 | COL | R | R795LT R802LT R804LT |
| --- | --- | --- | --- | --- | --- |

IEG347.Signature

| Digital signature See chapter "Digital signature". | - |  |  | 0..1 | R796LT R797LT R798LT |
| --- | --- | --- | --- | --- | --- |

## IEG348

PRESENTATION NOTIFICATION VALIDATION

A message of acceptance or rejection of presentation notification (or its amendment). It is sent by the Customs Office of Presentation to the Person Presenting the Goods.

When Rejection data exist then presentation notification (or its amendment) is rejected. Otherwise presentation notification (or its amendment) is accepted.

Structure

| IEG348 | 1 | - |
| --- | --- | --- |
| — Notification | 1 | - |
| — — Rejection | 0..1 | - |
| — — PersonPresentingGoods | 1 | - |
| — — CustomsOfficeOfPresentation | 1 | - |
| — Warning | 0..999 | - |
| — Error | 0..999 | R809LT |
| — Signature | 0..1 | - |

Contents

IEG348

| Root element of the message | - |  |  | 1 | - |
| --- | --- | --- | --- | --- | --- |

| preparationDateAndTime Preparation date and time of the message | - | dateTime | - | R | - |
| --- | --- | --- | --- | --- | --- |
| messageIdentification Message identification Unique identifier of the message assigned by sender. | - | an..40 | - | R | R783LT |
| correlationIdentifier Correlation Identifier Value of messageIdentification data item of the IEG347 message to which this message is direct response. | - | an..40 | - | R | R783LT |

IEG348.Notification

| Notification | - |  |  | 1 | - |
| --- | --- | --- | --- | --- | --- |

| lrn LRN Local Reference Number (LRN) of Presentation Notification. | 12 09 000 000 | an..22 | - | O | R791LT R800LT |
| --- | --- | --- | --- | --- | --- |
| mrn MRN Master Reference Number (MRN) of Presentation Notification. | - | an18 | - | O | R780LT R788LT R800LT |
| dateAndTimeOfPresentationOfGoods Date and time of presentation of the goods | 15 08 000 000 | dateTime | - | O | - |

IEG348.Notification.Rejection

| Rejection | - |  |  | 0..1 | - |
| --- | --- | --- | --- | --- | --- |

| rejectionDateAndTime Rejection date and time | - | dateTime | - | R | - |
| --- | --- | --- | --- | --- | --- |
| rejectionCode Rejection code | - | an..3 | 225NOV | R | - |
| rejectionReason Rejection reason | - | an..512 | - | O | R780LT R809LT |

IEG348.Notification.PersonPresentingGoods

| Person presenting the goods | 13 11 000 000 |  |  | 1 | - |
| --- | --- | --- | --- | --- | --- |

| identificationNumber Identification number | 13 11 017 000 | an..17 | - | R | R790LT R806LT |
| --- | --- | --- | --- | --- | --- |

IEG348.Notification.CustomsOfficeOfPresentation

| Customs office of presentation | 17 09 000 000 |  |  | 1 | - |
| --- | --- | --- | --- | --- | --- |

| referenceNumber Reference number | 17 09 001 000 | an8 | COL | R | R795LT R804LT |
| --- | --- | --- | --- | --- | --- |

IEG348.Warning

| Warning | - |  |  | 0..999 | - |
| --- | --- | --- | --- | --- | --- |

| sequenceNumber Sequence number | - | n..5 | - | R | R789LT |
| --- | --- | --- | --- | --- | --- |
| pointer Pointer Pointer to the place where warning occurred. | - | an..512 | - | R | R780LT |
| code Code Code of warning | - | an..2 | 45LT_49 | R | - |
| reason Reason Identifier of rule which produced warning. | - | an..10 | - | O | R780LT |
| originalAttribute Original attribute Original value the warning concerns. | - | an..512 | - | O | R780LT |
| description Description Warning description. | - | an..1024 | - | O | R780LT |
| source Source Code of warning source (i.e., code of system or process in the system). | - | an..20 | 243NOV | O | - |
| serious Serious Serious flag. Value '1' means that warning is serious. | - | boolean | - | O | - |

IEG348.Error

| Functional error | - |  |  | 0..999 | R809LT |
| --- | --- | --- | --- | --- | --- |

| sequenceNumber Sequence number | - | n..5 | - | R | R789LT |
| --- | --- | --- | --- | --- | --- |
| pointer Pointer Pointer to the place where error occurred. | - | an..512 | - | R | R780LT |
| code Code Code of error | - | an..2 | 45LT_49 | R | - |
| reason Reason Identifier of rule which produced error. | - | an..10 | - | O | R780LT |
| originalAttribute Original attribute Original value the error concerns. | - | an..512 | - | O | R780LT |
| description Description Error description. | - | an..1024 | - | O | R780LT |
| source Source Code of error source (i.e., code of system or process in the system). | - | an..20 | 243NOV | O | - |

IEG348.Signature

| Digital signature See chapter "Digital signature". | - |  |  | 0..1 | - |
| --- | --- | --- | --- | --- | --- |

## IEG349

PRESENTATION NOTIFICATION ITEMS REJECTION

This message is sent to the Person Presenting the Goods from Customs Office of Presentation to inform that pointed in IEG347 particular Entry Summary Declarations cannot be presented (e.g. goods does not exist).

Structure

| IEG349 | 1 | - |
| --- | --- | --- |
| — Notification | 1 | - |
| — — Declaration | 1..9999 | - |
| — — — Rejection | 1 | - |
| — — PersonPresentingGoods | 1 | - |
| — — CustomsOfficeOfPresentation | 1 | - |
| — Signature | 0..1 | - |

Contents

IEG349

| Root element of the message | - |  |  | 1 | - |
| --- | --- | --- | --- | --- | --- |

| preparationDateAndTime Preparation date and time of the message | - | dateTime | - | R | - |
| --- | --- | --- | --- | --- | --- |
| messageIdentification Message identification Unique identifier of the message assigned by sender. | - | an..40 | - | R | R783LT |

IEG349.Notification

| Notification | - |  |  | 1 | - |
| --- | --- | --- | --- | --- | --- |

| mrn MRN Master Reference Number (MRN) of Presentation Notification. | - | an18 | - | R | R780LT R788LT |
| --- | --- | --- | --- | --- | --- |

IEG349.Notification.Declaration

| Entry Summary Declaration Fxx declaration header data level | - |  |  | 1..9999 | - |
| --- | --- | --- | --- | --- | --- |

| mrn MRN Master Reference Number (MRN) | - | an18 | - | R | R780LT R788LT |
| --- | --- | --- | --- | --- | --- |

IEG349.Notification.Declaration.Rejection

| Rejection | - |  |  | 1 | - |
| --- | --- | --- | --- | --- | --- |

| rejectionDateAndTime Rejection date and time | - | dateTime | - | R | - |
| --- | --- | --- | --- | --- | --- |
| rejectionCode Rejection code | - | an..3 | 225NOV | R | - |
| rejectionReason Rejection reason | - | an..512 | - | O | R780LT R809LT |

IEG349.Notification.PersonPresentingGoods

| Person presenting the goods | 13 11 000 000 |  |  | 1 | - |
| --- | --- | --- | --- | --- | --- |

| identificationNumber Identification number | 13 11 017 000 | an..17 | - | R | R790LT R806LT |
| --- | --- | --- | --- | --- | --- |

IEG349.Notification.CustomsOfficeOfPresentation

| Customs office of presentation | 17 09 000 000 |  |  | 1 | - |
| --- | --- | --- | --- | --- | --- |

| referenceNumber Reference number | 17 09 001 000 | an8 | COL | R | R795LT R804LT |
| --- | --- | --- | --- | --- | --- |

IEG349.Signature

| Digital signature See chapter "Digital signature". | - |  |  | 0..1 | - |
| --- | --- | --- | --- | --- | --- |

## Rules

| R780LT | #SchemaValidated |
| --- | --- |
| Value of this data item (free text item) cannot: - be empty, - contain only white spaces (like spaces, tabulators), - contain white spaces at the beginning or at the end of the text. Value of this data item cannot be empty. |  |
| R781LT | #SchemaValidated |
| Value of this data item must be greater than 0. Value of this data item must be greater than zero. |  |
| R783LT | #SchemaValidated |
| messageIdentification data item cannot be empty and may contain only the following characters: '0-9', 'A-Z', 'a-z', '-'. Message identification data item cannot be empty and may contain only the following characters: '0-9', 'A-Z', 'a-z', '-'. |  |
| R784LT | #Contextual |
| messageIdentification data item must be unique among messages of the same type sent to the system by the same sender (identified by identificationNumber pointed by the messageSender data item). Message identification data item must be unique among messages of the same type sent to the system by the same sender (identified by identification number pointed by the message sender data item). |  |
| R785LT | #SchemaValidated |
| Possible XPath values of this data item are defined by enumerations in particular schema definitions e.g., /IEG347/Notification/PersonPresentingGoods/identificationNumber. Possible values of this data item are defined in particular schema definitions. |  |
| R786LT | #Contextual |
| identificationNumber pointed by messageSender has to be provided. Message sender must be declared. |  |
| R787LT | #Contextual |
| Notification.lrn of notification message together with message sender identificationNumber (see messageSender data item) cannot point to existing notification of the same type (i.e., notification LRN per sender and notification type must be unique). LRN number of notification must be unique. |  |
| R788LT |  |
| There must be a valid control digit in the mrn data item. The MRN must have a valid control digit. |  |
| R789LT |  |
| Each sequenceNumber is unique for the Data group it belongs to. The sequence numbers shall be sequential, starting from '1' for the first iteration of the Data group and increasing by '1' for each iteration. Sequence numbers must be unique sequential numbers in collection starting from 1. |  |
| R790LT | #SchemaValidated |
| EORI/TCUIN value should comply with the following pattern: [A-Z]{2}[\x21-\x7E]{1,15} It means: - first two chars from the ASCII table scope "A".."Z" - next chars (at least 1, maximum 15) from the ASCII table scope "!".."~" EORI/TCUIN value should comply with the following pattern: first two chars from "A".."Z" scope, next chars (at least 1, maximum 15) from the wider scope: "!".."~" |  |
| R791LT | #SchemaValidated |
| Declaration.lrn cannot be empty and may contain only the following characters: '0-9', 'A-Z', 'a-z', '-', '_', '.', ':', '/', '\' LRN cannot be empty and may contain only the following characters: '0-9', 'A-Z', 'a-z', '-', '_', '.', ':', '/', '\'. |  |
| R793LT |  |
| Pointed date/time must be earlier than or equal to current date/time. Pointed date/time must be earlier than or equal to current date/time. |  |
| R794LT | #SchemaValidated |
| containerIdentificationNumber may contain only the following characters: '0-9', 'A-Z', 'a-z'. Container number may only contain numbers from '0' to '9', capital letters from 'A' to 'Z' and lowercase letters from 'a' to 'z'. |  |
| R795LT | #SchemaValidated |
| First two characters of customs office referenceNumber can be only 'LT'. Only "LT" Lithuanian customs office is acceptable. |  |
| R796LT | #Contextual |
| The content of the message must be integral with the digital signature The content of the message must be integral with the digital signature. |  |
| R797LT | #Contextual |
| The person who signed the message must have the right to act for the sender of the message Remarks: - Person is identified by thumbprint of digital signature. - Message sender is pointed by messageSender data item of the root message element. - This rule is not relevant for messages sent from PPMKS Portal. The person who signed the message must be authorised in BAP (Common User Management Portal) to act on behalf of Declarant/Representative who sent the message. |  |
| R798LT | #Contextual |
| Signature is required for electronic communication A digital signature is required for electronic communication. |  |
| R800LT |  |
| IF Notification.mrn is declared THEN Notification.lrn cannot be used ELSE Notification.lrn is required When MRN is not provided (notification is sent the first time) then LRN is required. When MRN is provided (notification amendment case) then LRN cannot be used. |  |
| R801LT | #Contextual |
| IF Notification.lrn is provided THEN Notification.lrn (together with message sender identifier see messageSender data item) cannot point to existing and not rejected Presentation Notification ELSE IF Notification.mrn is provided Notification.mrn must point to existing notification which is not finalized yet LRN of particular notification type must be unique per notification sender. MRN must point to existing and not finalized notification. |  |
| R802LT | #Contextual |
| IF Notification.mrn is provided (amendment case) THEN the following data must be exactly the same as in pointed notification: - messageSender, - PersonPresentingGoods.identificationNumber - CustomsOfficeOfPresentation.referenceNumber The rest of message data will update pointed notification. The following message data, must be exactly the same as in pointed notification: - message sender, - identification number of person presenting goods, - reference number of presentation customs office. |  |
| R803LT |  |
| - Declaration.Consignment.TransportDocumentMasterLevel cannot be used - Declaration.Consignment.HouseConsignment with TransportDocumentHouseLevel are required During ICS2 Release 1 transport document cannot be used on consignment level. During ICS2 Release 1 transport document on house consignment level is required. |  |
| R804LT |  |
| Notification.CustomsOfficeOfPresentation.referenceNumber must exist in correlation code list 94LT (code1) with role (code2) equals to "9IC" (see code list 56). Customs office of presentation must have "presentation office (ICS2)" role. |  |
| R805LT |  |
| IF Consignment.HouseConsignment.TransportDocumentHouseLevel.type = "N703" (House air waybill), "N740" (Airway bill general) THEN Consignment.Carrier is required For transport documents: House air waybill (N703) or Airway bill general (N740) Carrier is required. |  |
| R806LT | #Contextual |
| Only EORI number is acceptable. Remark EORI is checked in MPR system. Only EORI number is acceptable. |  |
| R807LT |  |
| IF LocationOfGoods.qualifierOfIdentification = 'U' (UN/LOCODE) THEN LocationOfGoods.unLocode is required AND LocationOfGoods.authorisationNumber LocationOfGoods.CustomsOffice LocationOfGoods.GNSS LocationOfGoods.EconomicOperator LocationOfGoods.Address LocationOfGoods.PostcodeAddress cannot be used ELSE IF LocationOfGoods.qualifierOfIdentification = 'Y' (Authorisation number) THEN LocationOfGoods.authorisationNumber is required AND LocationOfGoods.unLocode LocationOfGoods.CustomsOffice LocationOfGoods.GNSS LocationOfGoods.EconomicOperator LocationOfGoods.Address LocationOfGoods.PostcodeAddress cannot be used ELSE IF LocationOfGoods.qualifierOfIdentification = 'V' (Customs office identifier) THEN LocationOfGoods.CustomsOffice is required AND LocationOfGoods.unLocode LocationOfGoods.authorisationNumber LocationOfGoods.GNSS LocationOfGoods.EconomicOperator LocationOfGoods.Address LocationOfGoods.PostcodeAddress LocationOfGoods.ContactPerson cannot be used ELSE IF LocationOfGoods.qualifierOfIdentification = 'W' (GNSS coordinates) THEN LocationOfGoods.GNSS is required AND LocationOfGoods.unLocode LocationOfGoods.authorisationNumber LocationOfGoods.CustomsOffice LocationOfGoods.EconomicOperator LocationOfGoods.Address LocationOfGoods.PostcodeAddress cannot be used ELSE IF LocationOfGoods.qualifierOfIdentification = 'X' (EORI number) THEN LocationOfGoods.EconomicOperator is required AND LocationOfGoods.unLocode LocationOfGoods.authorisationNumber LocationOfGoods.CustomsOffice LocationOfGoods.GNSS LocationOfGoods.Address LocationOfGoods.PostcodeAddress cannot be used ELSE IF LocationOfGoods.qualifierOfIdentification = 'Z' (Address) THEN LocationOfGoods.Address is required AND LocationOfGoods.unLocode LocationOfGoods.authorisationNumber LocationOfGoods.CustomsOffice LocationOfGoods.GNSS LocationOfGoods.EconomicOperator LocationOfGoods.PostcodeAddress cannot be used ELSE IF LocationOfGoods.qualifierOfIdentification = 'T' (Postal code) THEN LocationOfGoods.PostcodeAddress is required AND LocationOfGoods.unLocode LocationOfGoods.authorisationNumber LocationOfGoods.CustomsOffice LocationOfGoods.GNSS LocationOfGoods.EconomicOperator LocationOfGoods.Address cannot be used When location of goods by qualifier of identification points to UN/LOCODE then it is required and other location of goods data: authorisation number, customs office, GNSS coordinates, economic operator, address or postal address cannot be used. When location of goods by qualifier of identification points to authorisation number then it is required and other location of goods data: UN/LOCODE, customs office, GNSS coordinates, economic operator, address or postal address cannot be used. When location of goods by qualifier of identification points to customs office then it is required and other location of goods data: UN/LOCODE, authorisation number, GNSS coordinates, economic operator, address or postal address cannot be used. When location of goods by qualifier of identification points to GNSS coordinates then they are required and other location of goods data: UN/LOCODE, authorisation number, customs office, economic operator, address or postal address cannot be used. When location of goods by qualifier of identification points to economic operator then it is required and other location of goods data: UN/LOCODE, authorisation number, customs office, GNSS coordinates, address or postal address cannot be used. When location of goods by qualifier of identification points to address then it is required and other location of goods data: UN/LOCODE, authorisation number, customs office, GNSS coordinates, economic operator or postal address cannot be used. When location of goods by qualifier of identification points to postal address then it is required and other location of goods data: UN/LOCODE, authorisation number, customs office, GNSS coordinates, economic operator or address cannot be used. |  |
| R808LT |  |
| IF LocationOfGoods.EconomicOperator OR LocationOfGoods.authorisationNumber is provided THEN LocationOfGoods.additionalIdentifier can be used ELSE LocationOfGoods.additionalIdentifier cannot be used For location of goods other then managed by economic operator or defined by authorisation the additional identifier cannot be used. |  |
| R809LT |  |
| IF Rejection.rejectionCode = '999' (Other) THEN at least Rejection.rejectionReason OR Error (where relevant) is required When rejection code points "Other" (999) then at least rejection reason or error data are required. |  |